check to see how linear a speaker is as the bandwidth is

changed.  in other words, as the input bandwidth to the speaker

is changed, while the rms or dB SPL is held constant, how does

the output dB SPL change?

after having made an ear.cal file, switch over to a flat

microphone and run calib.bw.  stimuli can either be noise or

tone stacks.  result should be flat and be equal to the SPL

field in the dialogue box.  note that this can only be done with

a naturally flat mic (ie B&K), and not something which is just

software-calibrated with a .fr file (like the Knowles).

this module can also be used to check to make sure the code is

scaling things properly with bandwidth.