Plays sinusoidal out-of-phase amplitude-modulated stimuli in each ear

and, hence, sweep IIDs.  Similar in spirit to binaural beats as in

Moiseff & Konishi (1981).

Parameter fields:

'Carrier' and 'Signal' specify the carrier and signal frequencies

respectively.  So one ear gets Carrier+0.5*Signal and

the other Carrier-0.5*Signal.  Which ear is which is alternated

by trial.

Displayed is an iid tuning curve inferred from a periodhistogram

of period 1/Carrier.  Trials can be either lumped togethor, or

sorted by which ear had the higher frequency.

It's recommended to use long durations to minimize unwanted

effects from hysteresis and latency and artifacts due to the

rise/fall times.