Vary freq while keeping 'iid' and 'itd' fixed.  Stimulus can either be

monoaural left or right (L or R) or binaural (B).  At each frequency, abi

is adjusted until the quietest sound is found which elicits a response

significantly different from spontaneous.  The algorithm is a binary

search starting with 'High' and 'Low' intensities.  Termination occurrs

when the difference between the upper and lower boundaries is

'Resolution'.  The decision regarding which boundary to move is done by

comparing the current response against spontaneous with a permutation test

(Siegel and Castellan, 1988).  'p-val' specifies the cutoff.  The results

display both the upper and lower boundaries.  If the initial 'High' is not

above spontaneous, or the initial 'Low' is not below spontaneous, a

warning is given.