WINDOW DISCRIMINATOR

when mode is set to 'window' in the config menu, a software-implemented

window disriminator is used to count spikes.  the algorithm is as follows:

all events which exceed a given threshold are potential spikes.  from the

potential spikes, good spikes are chosen by their waveforms' relationship to

three optional dots.  dots are either red or green.  the waveforms of good

spikes go below red dots and above green dots.  if no dots are specified,

then all potential spikes are good.

the parameters of the discriminator can be adjusted via the keyboard when

the cursor is in the box which displays the spike waveforms.  to change the

threshold for potential spikes, first press 't', and then use the 'k' and

'j' keys to move it up and down, respectively.  initially, no dots are on.

to turn a dot on, first select it as the current dot by pressing '1', '2',

or '3'.  then toggle the on/off state by pressing 'o'.  to change the color

press 'c'.  the position of the dot can be changed with vi-like key

bindings: 'h' is left, 'l' is right, 'j' is down, and 'k' is up.  to reset

parameters to their factory default settings use 'r' for just the current

dot or 'a' for all the dots plus the threshold.

good spikes are displayed in black and bad spikes in gray.  all the peaks

are aligned in time on the vertical blue line.  peaks are further aligned in

voltage such that they overlap at the top of the window.  the alignment in

time and voltage makes it easier to judge the similarity in the shape of the

waveforms.  the alignment in voltage, however, makes it difficult to plot

the threshold for spikes, as low-frequency noise will cause the peaks to be

varying amounts above threshold.  this problem is resolved by plotting two

horizontal dashed blue lines representing the minimum and maximum amounts

that the good spikes are above threshold.

SOUNDTYPES

nosnd.............. Silence

0.................. broadband white noise

bb................. broadband white noise

hp................. high pass white noise (4-13kHz)

bp=<lo>-<hi> ...... band pass white noise from <lo> Hz to <hi> Hz

[<lo>-<hi>] ....... band pass white noise from <lo> Hz to <hi> Hz

CF=<cf>,BW=<w> .... bandpass white noise from cf-(bw/2) Hz to cf+(bw/2) Hz

{<cf>,<w>} .......  bandpass white noise from cf-(bw/2) Hz to cf+(bw/2) Hz

tone=<freq>........ pure tone of <freq> Hz

<freq>............. pure tone of <freq> Hz

stack=<range>...... pure tone list (tone stack)

(see RANGES below for description of different

range styles)

fmsweep=<lo>-<hi>:<n>    FM sweep from <lo> Hz to <hi> Hz in <n> steps

bc=<percent>....... binaurally correlated noise: correlation coefficient

between left and right channels will be <percent>

NOTE: bc sounds are played UNCALIBRATED

click=<freq>....... click: one cycle of a tone of frequency <freq>

NOTE: clicks are played UNCALIBRATED

RANGES

they describe the sequence of stimuli you want to play and consist of

comma separated arguments.  each argument can be a single number, or

something of one of the following forms

#1:#2:#3   go from #1 to #2 in steps of #3

#1:#2:/#3  go from #1 to #2 with #3 steps spaced linearly

#1:#2:@#3  go from #1 to #2 with #3 steps spaced exponentially

examples

-30,-20,-10,0,10,20,30 yields -30,-20,-10,0,10,20,30

-30:30:10 yields -30,-20,-10,0,10,20,30

-30:30:10,-5,5 yields -30,-20,-10,-5,0,5,10,20,30

1000:10000:/10 yields 1000,2000,3000,4000,5000,6000,7000,8000,9000,10000

1000:10000:@10 yields 1000,1292,1668,2154,2783,3594,4642 5995,7743,10000

the following notation can be used to shift entire ranges

(...)+#1

(...)-#1

the ellipses denote the range (a comma separated list as above), and #1

indicates how much to shift everything by.

the stimuli are randomized *if* the Randomize variable is set in Prefs.

APPENDING

a datafile can be appended with either more repetitions of the same

range or an equal number of repetitions of a different range *if* all

other stimulus parameters (sound spec, abi, duration, etc) are the

same.  simply choose the file to be appended when asked for a

filename and it will ask if you want to append it.

example:  you take an IID curve with -40:40:5 and 10 reps, decide

that the error bars are too big and want to take more reps.  so just

press start again, choose the same file, and click on the append

button when it prompts you.  voila, now you have 20 reps.  now lets

say you want to increase the resolution of the data near the peak in

this same file.  the peak is around -5 dB, so you change the range

box to read -7,-3 and change the reps box to read 20 (it must match

the number of reps already in the file).  then click on start, choose

the same file, click on append, and voila, now you have 20 reps of

-40:40:5,-7,-3.

CHANGING ANIMALS

xdphys saves all the environment variables (ie all the entries in the

various worksheets it has, eg Prefs, Config, etc.) in statefiles.

there are two types of statefiles: one is called ~/.xdphysrc/config.xdphys

the other is called ~/.xdphysrc/xd_*, where '*' is the number of the

current animal as specified in Unit,Pos/Animal.  both types are saved at

many different times including upon quitting, executing File/Update,

and executing anything in the New menu.  when xdphys is started, it is

config.xdphys that is initially loaded.  so all the variables, including

Unit,Pos/Animal will be the same as last time.  if you're working on

a different animal this time, simply go to New/ChangeOwl and choose the

xd_* statefile of the animal you're currently using.  if it's a new animal

this file doesn't exist, so you'll have to go to Unit,Pos and change

Animal.  then execute File/Update and a new xd_* file will be created

(alternately, you could just wait until you quit and it will do it

for you).

SPONTANEOUS STIMULI

there is a new flag in the Prefs menu called Spont_Stims.  if this is 1,

then spontaneous trials will be automatically included in each

repetition and the spontaneous rate will be computed from duration part

of the epoch in each of these trials.  if it is 0, then no such trials

will be included and the spontaneous rate will be computed from the

delay portion of all the trials.  if you're really concerned about

maximizing how much data you can take in a given period of time, you

should consider setting Spont_Stims to 0.

HOT KEYS

hot keys are keyboard shortcuts to access menu commands.  currently, the

following have been implemented.

<meta>u   new unit

<meta>q   quit

pause     pause

there are also, of course, all the hot keys that the rig module uses.

they are documented in the rigs docs.