THE PREFS WINDOW

================

-------------------------------------

The Numbered Buttons

-------------------------------------

There are three buttons, numbered "1", "2" and "3" at the top of the Prefs

window.  Clicking on each button will load a different "page" of Prefs.

You can use these three "pages" to store different sets of Prefs for

different kinds of experiments.  The Prefs on the page corresponding to

the button that is selected when the Prefs window is closed will be

the set of Prefs used in following trials.

-------------------------------------

Stimulus Parameters

-------------------------------------

ISI            Desired interstimulus interval.  The effective ISI may

be higher than this: it will, at least, be greater than

the Epoch, due to time needed to save data, and create the

next stimulus.

Reps           Number of times each stimulus will be played.

Delay          Time, measured from the start of the epoch, at which the

stimulus starts (milliseconds).

Dur            The duration of the stimulus (milliseconds)

Epoch          The length of the trial (milliseconds).  This must be

greater than or equal to Delay+Dur.

Rise           Rise time for the stimulus envelope.  Sound intensity

will rise from 0 to maximum linearly over this time period.

(milliseconds)

Fall           Fall time for the stimulus envelope.  Sound intensity

will fall from maximum intensity to 0 linearly over this

time period.  (milliseconds)

Spont_Stims    If 0, spontaneous rate data will be calculated from data

taken within the period between 0 and (Delay) ms for

each trial.  If 1, separate trials with no stimulus will

be recorded.

Randomize      Randomize stimuli (=1) or not (=0)

low, high      specifies the low and high pass limits of the 'bb' or

'0' stimulus specification.

rad_vary       change the starting phase from trial to trial (=1) or

keep it the same across trials (=0)

rms            scale stacks to keep the total rms constant independent

of the number of tones (=1), or keep the power in each

band constant (=0).

cal_file       specifies the calibration file to use.  note that

changes in this parameter will not take effect until

either xdphys is restarted or File/Reload Cal is done.

use_cache      for each unique combination of itd and bc, generate a

single noise stimulus twice as long as needed, cache it,

and use subsets of it on each rep (=1) or don't use

cached copies; rather, generate a novel noise stimulus

each time (=0).  note that changes in this parameter

will not take effect until xdphys is restarted.

-------------------------------------

Analog Save Parameters

-------------------------------------

channel1       Set to "on" to save analog data from this channel to the

data file.  Set it to "off" to not save analog data from

this channel.

channel2       Set to "on" to save analog data from this channel to the

data file.  Set it to "off" to not save analog data from

this channel.

ana-decimate   Downsample analog data by this factor.

ana-every      If ana-every is set to "N", save every analog waveforms

for every "N"th trial.

---------------------------------

Discrimination Settings

---------------------------------

discrim_channel   Set discrim_channel to the ID of the AD channel which is

tracer_channel    being used to discriminate spikes.  Set tracer_channel to

the channel which has the analog signal from the electrode

as its input.  tracer_channel is used by the tracer

widget (the lower third of the tracer window).

o  If mode == ttl, and the SD1/ET1 is being used to

discriminate spikes, discrim_channel and tracer_channel

should be set to the same channel, and the input to this

channel should be a duplicate of the input to the SD1/ET1.

o  If mode == ttl, and a hardware discriminator which

outputs TTL pulses is being used, discrim_channel should

be set to the channel which has the TTL output of

the discriminator as its input, and tracer_channel

should be set to the channel which has the analog

signal from the electrode as its input.

o  If mode == window, discrim_channel and tracer_channel

should be set to the same channel, and the input to this

channel should be the analog signal from the electrode.

mode             Specify how spikes will be collected:

ttl:    Use SD1/ET1 (if present), or TTL output of a

hardware discrimintor to discriminate spikes.

window: Use a software-based window discriminator to

discriminate spikes.  (see below for detailed

explanation of the operation of this discriminator)

ss:     Don't use it, it doesn't work.

jump             (window only) Specifies the minimum step size for moving

the upper and lower level discriminators in the

software-based window discriminator.

---------------------------------

Window Discriminator Operation

---------------------------------

NOTE: See xdphys main help (click "Help" on the main xdphys window) for

a more thorough explanation.

The software-based window discriminator mimics a hardware discriminator

in that there are lower and upper thresholds.  Spikes are triggered when

the voltage exceeds the lower threshold, but does not exceed the

optional upper threshold.  This software-based discriminator has an

additional feature which most hardware-based discriminators don't have:

a time window, which is optional.  The voltage must drop below the

lower threshold within a specified time.  While the cursor is in the

lower third of the xdphys window, the following keypad keys manipulate

the discrimnator

+-----+------+-----+-----+

|     |      |     |     |

+-----+------+-----+-----+

|     |      |     |     |   l = lower level

+-----+------+-----+-----+   u = upper levels

| l++ |  w++ | u++ |     |   w = both lower and upper levels

+-----+------+-----+     |   t = time window

| t-- | zero | t++ |     |   ++ = increase

+-----+------+-----+-----+   -- = decrease

| l-- |  w-- | u-- |     |

+-----+------+-----+     |

|            |     |     |

+------------+-----+-----+

On the regular keyboard, 'u' toggles the optional upper level on and off,

and 't'  toggles the optional time window on and off.  The '5' on the

keypad, labelled zero above, puts the lower level at the average DC value

of the input voltage.

-------------------------------------

Rig Controls

-------------------------------------

auto_shutoff  The rig will stop playback after this many stimuli have

been presented.

itd++         In the rig, increment or decrement the ITD of the stimulus

by this many microseconds for each ITD change keypress.

iid++         In the rig, increment or decrement the IID of the stimulus

by this many dB for each IID change keypress.

freq++        In the rig, increment or decrement the frequency of the stimulus

by this many Hz for each frequency change keypress.

-------------------------------------

Trace Displays

-------------------------------------

These parameters refer to the "tracer" graph: the graph at the bottom

of the main xdphys window, in which the waveforms for each detected spike

are displayed.

msPre        The start of the waveforms displayed in the "tracer" graph

begin msPre milliseconds before the corresponding discriminated

spike time.

msPost       The end of the waveforms displayed in the "tracer" graph

begin msPost milliseconds after the corresponding discriminated

spike time

maxTraces    At most maxTraces waveforms will be displayed in the tracer per

trial.

autoscale    If 1, make the waveforms fill the full vertical scale of the

"tracer" graph.

---------------------------------

Histogram Windows

---------------------------------

psth_bin       Bin size for the PSTH graph, in milliseconds

isih_bin       Bin size for the ISIH graph, in milliseconds

isih_max       The maximum value on the x-axis of the ISIH graph

decay          Specifies the exponential decay rate of the histograms

in the upper third of the xdphys window.  These histograms

are only decayed during rig operation, and not during execution

of any other module.

-------------------------------------

LED Parameters

-------------------------------------

If you are not Dr. Jose Luis Pena, you do not need this section:

set use_led to 0. If you are him, read on.

use_led        Set to 1 to enable use of the LED optic tectum stimulator.

led_delay      Time, measured from the start of the epoch, at which the

at which the LED(s) selected on the LED stimulator will

illuminate (milliseconds).

led_dur        The length of time after led_delay for which the LED(s)

will stay illuminated.