calibrate a speaker to a given environment using a microphone that

has been software-calibrated.  the resulting file can be used by

xdphys to calibrate the magnitude and phase of a sound.  if intended

for such use, it should be called ear.cal, and put in the same

directory in which xdphys is started.

the files used to calibrate the mic are generated by calib.fr and are

specified in the Prefs dialogue box, lines labelled Mic_Adjust_[Left,

Right].  if left blank, the defaults are $XDPHYSDIR/calib/$HOST.k[left,

right].fr.  the So[L,R] fields are ignored as that information is

contained in the .fr files specified in the Mic_Adjust[Left,Right]

fields.

to keep sounds above the noise floor and below the threshold of pain,

the attenuators are not kept fixed, but changed from trial to trial

to always maintain the sound between max_spl and min_spl.  the initial

attenuator setting is atten, and the amount which the attenuators

are changed if clipping occurrs is auto_step.

to check to see how well the system can calibrate sounds, first

create a .cal file with check set to 0.  then specify this .cal

file in check_file and set check to either PT, TS, or WN, to check

with pure tones, tone stacks, or white noise, respectively.  in each

case, a narrow band stimulus is created by the same code that

xdphys uses to create each of these types of stimuli.