The click module is used for presenting a series

of binaural rarefaction or condensation clicks while

varying ITD.

The stimulus is a step function of maximum (for

condensation) or minimum (for rarefaction) voltage

lasting for <length> samples.

EXPLANATION OF FIELDS

itd_range

The range of ITDs to present.  See the RANGE

section of the help document accessed through

the "Help" button on the main window for an

explanation of how to define ranges.

NOTE: ITDs are generated by advancing the

click in either the left or right channel

by an integral number of samples.  As such,

achievable ITDs are only those which are

evenly divisible by the sampling period.

For requested ITDs which are not evenly

divisible by the sampling period, the

closest achievable ITD is used instead.

length

This is the length of the click in samples.

type

Type can be either 0 or 1.

If 0 is selected, a rarefaction click will be

played.

If 1 is selected, a condensation click will be

played.

left_atten

right_atten

The appropriate PA4s will be set to the

attenuation level given in these fields.

Side

Play monaural left (L), monaural right (R) or

binaural (B) sounds.