Exercise 9:	Plant Wide Process Control Design

Steady State Screening Plant Wide

Control Systems

I.  OBJECTIVE

The objective of this exercise is to demonstrate how the steady state gain matrix for a process can be used to screen plantwide control alternative architectures.  The process gain matrix is used to assess the effect of step disturbances, IDV(2) to IDV(7) on the product flow and composition for four control architectures.  The results of the exercise indicate both the viability of the architectures, and the importance of each disturbance.

II.  CONTROL TECHNOLOGY

Calculation of the raw steady state gain matrix, including integrating liquid levels, for the Tennessee Eastman process gives a matrix that is 41x12.  One of the ways that this matrix can be used to assess a plantwide control architecture involves making a plot of the key product variables versus the magnitude of a step disturbance.  In carrying out the calculations all variables except the product flow and composition are controlled.  A typical plot is illustrated below:

Figure 1

As illustrated architecture 1 produces smaller changes in the product flow and composition than architecture 2.  Thus, for the specific disturbance considered architecture 1 is better.  One can interpret the results in Fig. 1 as follows.  The underlying plantwide architecture 1 (exclusive of product variable control) does the best job in terms of eliminating the effect of the disturbance.  Thus, when the product variables are controlled, architecture 1 makes the resulting control problem easier than architecture 2.  Clearly, one could run into cases where one architecture is better for one disturbance and another for another disturbance.  In such cases tradeoffs would have to be made.  Also, one could make judgments about plantwide architectures based on manipulative variable movement, but this point is not treated in this exercise.

To calculate the results in Fig. 1 one can proceed as follows.  The gain matrix can be written as:

(1)

where

yc = measurements controlled (equal 0)

y  = measurements not controlled

u  = manipulated variables used

un = manipulated variables not used (equal 0)

d1 = disturbance

d2 = disturbance

Equation 1 can be solved for u as:

(2)

Setting un equal to 0. and substituting for u, one can solve for y as:

(3)

Equation 3 can be used to calculate changes in the product variables for a given disturbance.  The d1 and d2 vectors give the effects of a particular step disturbance on the measurements.  These vectors can be calculated using numerical differentiation and the same approach that was used to calculate the process gain matrix.  In the Tennessee Eastman process the first seven upsets, IDV(1) to IDV(7), involve steps and each can be analyzed.  Disturbance IDV(1) was considered in lecture.  In this exercise disturbances IDV(2) to IDV(7) are considered.

III.  PROCESS DESCRIPTION

A description of the Tennessee Eastman process has been given earlier.  In this exercise 4 plantwide control architectures are considered.  The first involves manipulating the following variables: E Feed, A Feed, Purge, Separator Exit Flow, Product Flow, and Reactor Cooling Water Valve.  The controlled variables are: Reactor Pressure and Level, Separator Level, Stripper Level, Reactor Feed %A, and Purge %B.  In architecture 2 the Reactor Temperature is controlled instead of the Reactor Pressure.  In Architectures 3 and 4 the same variables are controlled as architectures 1 and 2.  In architectures 3 and 4 the Condenser Cooling Water is used instead of the E Feed.

IV.  COMPUTER EXERCISE

The m file TEdis is used in this exercise.  To run this m file simply type TEdis and respond to the prompts for inputs.  Outputs are generated both as plots and as numerical values for the full step upsets.  For disturbances IDV(2) to IDV(7) calculate the effect of each disturbance.  Tabulate how much the Product Flow, %H in the Product, Reactor Temperature, and Reactor Pressure vary for each disturbance.  The form on the following page can be used for the tabulation.  For IDV(7) calculate the changes with and without the C Feed flow controller being operational.  A total of 7 cases should be run for each architecture.  The magnitude of the changes in the reactor and product variables can be used to rank not only the various architecture, but the importance of disturbances as well.

V.  RESULTS ANALYSIS

Which architecture performs the best for disturbances IDV(2) to IDV(7)?  For the Tennessee Eastman process does one have to consider any tradeoffs, i.e. one architecture is best for one upset bur poorer for another.  Which step disturbances are the hardest to control and which are the easiest?  Does the closure of the C Feed flow controller help with IDV(7)?  Based on your results do you recommend controlling Reactor Temperature or Pressure?  Based on your results should the E Feed or the Condenser Cooling Water be used for Reactor Level control?

|  |  | Architecture 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IDV |  React. P. kPa |  React. T. oC |  %H Prod. % |  |  Product (m3/hr) |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 C Feed On |  |  |  |  |  |  |
| 7 C Feed Off |  |  |  |  |  |  |

|  |  | Architecture 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IDV |  React. P. kPa |  React. T. oC |  %H Prod. % |  |  Product (m3/hr) |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 C Feed On |  |  |  |  |  |  |
| 7 C Feed Off |  |  |  |  |  |  |

|  |  | Architecture 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IDV |  React. P. kPa |  React. T. oC |  %H Prod. % |  |  Product (m3/hr) |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 C Feed On |  |  |  |  |  |  |
| 7 C Feed Off |  |  |  |  |  |  |

|  |  | Architecture 4 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IDV |  React. P. kPa |  React. T. oC |  %H Prod. % |  |  Product (m3/hr) |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 C Feed On |  |  |  |  |  |  |
| 7 C Feed Off |  |  |  |  |  |  |